Native American Drone Flute

Bill of Materials & Build Method

Set of 4 (F#m | Em | Dm | Am)  -  Walnut + Maple inlay  -  ShopBot CNC + lathe + Epilog laser

# 1. Project Summary

This document covers the materials, tools, and step-by-step build method for a set of four Native American drone flutes (Am, F#m, Em, Dm) inspired by the elementalflutes.com gallery, with broinwood-style geometric inlay across all eight facets and a feature panel on one facet of each flute. Construction is hybrid: split-body CNC for the interior chambers, lathe rounding, then CNC facet pass for the exterior. Each flute ships with a tunable drone block set (3 swappable blocks: unison, fifth, octave).

Estimated build time: 60-80 shop hours across 4-6 weeks (acclimation and finish cure dominate calendar time).

Estimated material cost: $516. Tools and bits are largely already on hand at Maker Nexus and Tony's home shop.

# 2. Bill of Materials

Quantities are for the complete set of 4 flutes plus a 25 percent waste/test factor. Pricing is approximate as of mid-2026.

## 2.1 Wood Stock

| # | Item | Qty | Spec | Source | Cost |
| --- | --- | --- | --- | --- | --- |
| 1.1 | Body blank | 4 | Black walnut, S4S, 1.5x1.5x32 in | Hardwoods to Get / local | $22 ea = $88 |
| 1.2 | Body blank (Em, Dm) | 2 | Black walnut, S4S, 1.75x1.75x36 in | Hardwoods to Get / local | $30 ea = $60 (replaces 1.1 for the 2 lower keys) |
| 1.3 | Inlay stock - bands | 2 | Hard maple, 1/16x2x24 in | Inventables / Bell Forest | $8 ea = $16 |
| 1.4 | Inlay stock - feature panel | 1 | Hard maple, 1/8x4x12 in (figured if available) | Bell Forest | $14 |
| 1.5 | Bird block stock | 1 | Black walnut, 1x1x10 in (cut to 4 blocks) | Scrap or 1.1 offcut | $0 (offcut) |
| 1.6 | Drone block stock | 1 | Black walnut, 1.75x1.75x18 in (cut to 12 blocks: 3 per flute) | Hardwoods to Get | $15 |

Subtotal: $193

## 2.2 Hardware & Finish

| # | Item | Qty | Spec | Source | Cost |
| --- | --- | --- | --- | --- | --- |
| 2.1 | Leather lacing for bird tie | 4 ft | Natural latigo, 3/16 width | Tandy Leather | $6 |
| 2.2 | Sealer/finish - Tru-Oil | 1 bottle | 3 oz | Brownells / Amazon | $10 |
| 2.3 | Carnauba wax | 1 tin | Howards Feed-N-Wax 16 oz | Hardware store | $10 |
| 2.4 | Painter tape, 1 in blue | 1 roll | 3M ScotchBlue | Hardware store | $6 |
| 2.5 | Sandpaper assortment | 1 set | 120, 180, 220, 320, 400, 600 grit | Hardware store | $15 |
| 2.6 | Wood glue | 1 bottle | Titebond II Premium 16 oz | Hardware store | $8 |
| 2.7 | CA glue (thin) | 1 bottle | Starbond EM-02 Super Fast Thin 2 oz | Starbond | $10 |
| 2.8 | CA accelerator | 1 bottle | Starbond NCF Quick Set Accelerator | Starbond | $8 |
| 2.9 | Foam tipped applicators | 20 ct | For Tru-Oil application | Amazon | $5 |

Subtotal: $78

## 2.3 CNC Tooling & Drill Bits

| # | Item | Qty | Spec | Source | Cost |
| --- | --- | --- | --- | --- | --- |
| 3.1 | CNC tooling - 1/16 upcut spiral | 1 | Carbide, 1/8 shank, 1/2 LOC | IDC Woodcraft | $22 |
| 3.2 | CNC tooling - 1/4 downcut spiral | 1 | Carbide, 1/4 shank, 1 LOC | IDC Woodcraft | $32 |
| 3.3 | CNC tooling - 1/2 downcut spiral | 1 | Carbide, 1/2 shank, 1.25 LOC | IDC Woodcraft | $45 |
| 3.4 | CNC tooling - 3/4 ball end | 1 | Carbide, 1/2 shank, 1.5 LOC | IDC Woodcraft | $58 |
| 3.5 | Lathe tools | use existing | Roughing gouge, skew, parting tool | Tony has these | - |
| 3.6 | Drill bits for tone holes | 4 | 1/4, 9/32, 5/16, 11/32 brad-point | Lee Valley | $22 |
| 3.7 | Reamer for SAC channel | 1 | 7/8 long-spur saw bit | Lee Valley | $18 |
| 3.8 | Forstner bits (drone socket) | 4 | 5/8, 3/4, 7/8, 1 in | Lee Valley | $48 |

Subtotal: $245 (one-time tooling)

## 2.4 CAD/CAM Files

| # | Item | Qty | Spec | Source | Cost |
| --- | --- | --- | --- | --- | --- |
| 4.1 | CNC G-code - bore profile pass | 4 files | One per key. Generated from SolidWorks via Vectric V-Carve. | Tony | - |
| 4.2 | CNC G-code - facet pass | 4 files | One per key. Octagon facet cuts. | Tony | - |
| 4.3 | CNC G-code - inlay pocket | 12 files | 3 bands x 4 keys. | Tony | - |
| 4.4 | CNC G-code - inlay plug | 12 files | Mating plugs cut from 1/16 maple. | Tony | - |
| 4.5 | CNC G-code - feature panel | 4 files | Pocket and plug pair per key. | Tony | - |
| 4.6 | CNC G-code - finger holes | 4 files | Six holes per flute, drilled or routed. | Tony | - |
| 4.7 | CNC G-code - drone block tenon | 4 files | Lathe-turned alternative possible. | Tony | - |

Files only - no dollar cost

## 2.5 Total

Total estimated material + tooling cost for the set: $516 + tooling already on hand

# 3. Tools & Equipment Inventory

Maker Nexus equipment (in the South Bay):

- ShopBot CNC router (4x8 bed)

- Wood lathe (12 in swing minimum, 36 in between centers)

- Epilog laser cutter (60W or 75W)

- Drill press

- Bandsaw, jointer, planer, table saw, drum sander

Tony's home shop:

- SolidWorks (parametric CAD)

- Vectric V-Carve (CAM for ShopBot)

- Hand tools (chisels, files, scrapers, calipers)

- Chromatic tuner (Korg or smartphone app)

# 4. CNC Toolpath Quick Reference

Recommended speeds and feeds. Tony will tune in the shop based on bit wear and spoilboard condition.

| Phase | Operation | Bit | Spindle (rpm) | Feed (ipm) | Depth/Pass | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | Bore profile | 1/4 downcut | 18000 | 80 | 0.250 | 2 passes max |
| 2 | SAC pocket | 1/4 downcut | 18000 | 80 | 0.250 | 2 passes |
| 2 | Splitting edge wall | 1/16 upcut | 20000 | 40 | 0.060 | Climb cut, single pass |
| 5 | Facet flattening | 1/2 downcut | 16000 | 120 | 0.080 | 8 facets x 1 pass each |
| 8 | Inlay band pocket | 1/16 upcut | 18000 | 60 | 0.060 | Plunge 20 ipm |
| 8 | Inlay plug profile | 1/16 upcut | 18000 | 60 | 0.030 | Plug stock = 1/16 maple |
| 8 | Feature panel pocket | 1/16 upcut | 18000 | 60 | 0.080 | Single pass |
| 6 | Tone hole drill | brad-point | drill press 1500 | manual | thru | Backer dowel inside bore |

Speeds and feeds are starting points. Verify on a scrap blank before committing.

# 5. Build Method

The build is organized into 13 phases, executed in order. Each phase has explicit steps with the relevant Excel/SolidWorks references baked in.

Phase 0: Design & CAD Prep

- Confirm A=440 reference; if alternate tuning desired, edit Master_Inputs!B9 in Excel and re-export.

- Open Drone-Flutes-Design.xlsx. Verify each key's row in Design_Table looks correct (length, bore, drone bore).

- Open DroneFlute_Master.SLDPRT in SolidWorks. Update from external reference (Excel) if prompted.

- Switch through configurations Am/F#m/Em/Dm to confirm geometry rebuilds without errors.

- Save As DXF: each facet face, each inlay sketch, the window cutout, and the hole-center sketch. Save to /Drone-Flutes/inlay-patterns/{key}/ folders.

- Generate G-code in V-Carve. Use the bit list in BOM section 3 and the toolpath table below.

- Sanity-check G-code in V-Carve preview before sending to ShopBot.

Phase 1: Stock Prep

- Mill all walnut blanks to S4S (squared four sides). Final blank dimensions per key from Design_Table!blank_thickness and blank_length.

- Joint one face flat and joint one edge square. Plane to thickness. Rip to width. Cut to length +1.5 in for end-grain trim.

- Inspect grain orientation: long grain MUST run parallel to the bore axis. Reject any blank with end-grain runout exceeding 1:12.

- Mark center reference line on every blank with a marking gauge or laser pointer along the long axis.

- Drill three datum holes (3/16) on each blank for CNC alignment pins. Locations: 1 in from each end and one in the middle.

- Acclimate blanks in Tony's shop for at least 7 days before the next phase.

Phase 2: Split-Body CNC (Bore + SAC)

- Resaw each blank exactly down the center using the bandsaw. Save the saw kerf - clean both halves with a hand plane to glue-ready surfaces.

- Mark each pair (left half / right half) with the same key letter so they don't get mixed.

- Set up CNC with 1/4 downcut spiral. Load the bore-profile G-code for the active key.

- Mount one half on the spoilboard with vacuum + datum pins. Run the bore-profile pass: 1/4 round-bottom channel = bore_ID/2 deep. Two passes recommended.

- Run the SAC pocket: same depth, but with the SAC_length and SAC_wall geometry from the Excel sheet baked into the G-code.

- Run the window/splitting-edge profile cut. Use 1/16 upcut for the splitting edge to leave a clean knife-thin wall.

- Flip the second half onto the same fixture (alignment pins do their job here). Run the same G-code mirrored.

- Repeat for all 4 keys (8 halves total).

Phase 3: Glue-Up

- Dry-fit each pair. Ensure the bore aligns cleanly when the halves meet. If not, sand mating faces lightly on a granite plate.

- Apply Titebond II to both mating faces. Use a thin, even coat.

- Clamp with at least 6 clamps along the length, alternating top and bottom. Place wax paper between clamps and the body to keep glue squeeze-out from sticking.

- Use the 3/16 dowels in the datum holes during glue-up to keep alignment perfect.

- Wait 24 hours minimum before unclamping.

- Inspect the glue line. If any visible gap, soak that area with thin CA + accelerator and re-clamp briefly.

Phase 4: Lathe Rounding

- Mount each glued blank between centers on the lathe. Drive center on the foot end, live center on the SAC end.

- Rough-turn to round at slow RPM (about 800). Use a 3/4 roughing gouge.

- Final-turn to body_OD_round (Excel Design_Table). Leave 0.05 in extra (the facet_allowance) - this is removed by faceting.

- Sand on the lathe through 220 grit while the body is true and concentric.

- Mark a reference line along the entire length with a pencil while the lathe spins slowly. This is the seam datum for the next phase.

Phase 5: CNC Faceting

- Build a V-block jig on the CNC bed: two pieces of MDF cut at 22.5 degrees each, plus end-stops to hold the body in axial position.

- For octagon (8 facets), each pass orients the body so one facet is parallel to the spoilboard.

- Load the 1/2 downcut spiral for facet flattening.

- Run pass 1 to remove facet_allowance + a tiny bit more (0.005 in safety) to make the top facet dead flat.

- Rotate the body 45 degrees in the V-block. Index using a pin or notch on the foot end.

- Run pass 2. Repeat for all 8 facets.

- Verify with calipers: body_OD_flat should match the Excel value within 0.010 in.

Phase 6: Tone Hole Drilling

- Mount the body in a horizontal jig with the top facet (where holes will be drilled) facing up.

- Set up a Forstner-style brad-point bit in the drill press. Use the diameter listed for each hole position (Hole_Schedule input cell).

- Mark hole centers using positions from the Hole_Schedule sheet, measured from the FOOT.

- Drill each hole carefully. Use a backer board inside the bore to prevent blowout (a piece of dowel slid into the bore).

- After drilling, sand each hole interior with 320-grit rolled sandpaper to remove fuzz.

- Tuning step: blow each hole open and use a chromatic tuner. Adjust by enlarging slightly with a chainsaw file or tapered reamer.

- Once tuned, ease the inside top edge of each hole at a 30-degree angle - this is the undercut and gives the flute its character.

Phase 7: Window & Splitting Edge

- Position the window cutout above the SAC-to-bore wall, on the top facet.

- Use a 1/16 upcut to plunge a starting hole at the splitting-edge location.

- Cut the window perimeter using the window_length x window_width sketch from SolidWorks.

- Carefully shape the splitting edge with a small chisel and 320-grit sandpaper. The edge should be knife-sharp.

- Test by gently puffing into the SAC end - you should hear a clean tone. If breathy, file the splitting edge sharper. If too thin, glue a small shim of veneer.

Phase 8: CNC Inlay (Pocket + Plug)

- Reorient the body so each facet can be CNC-pocketed. Use the V-block jig from Phase 5.

- Load the 1/16 upcut spiral. Set spindle to 18000 rpm, feed to 60 ipm, plunge to 20 ipm.

- Run pocket G-code for band1, band2, band3 on each facet of the body. Depth = 0.060 in.

- Switch to the maple inlay stock on a separate fixture. Run the matching plug G-code with 0.002 in oversize.

- Test fit. If the plug binds, sand its underside lightly. If it falls in, the pocket was cut wide - use thicker CA to compensate.

- Apply thin CA glue, press the plug into the pocket. Wait 20 seconds for cure.

- Repeat for the feature panel (one chosen facet, between band 2 and band 3): pocket + plug, depth 0.080 in.

- After all inlay is in place, surface-sand the body flat using a flat block and 220 grit. The plugs should sit flush.

Phase 9: Drone Block Set

- Cut three drone block blanks per flute (12 total): unison, fifth, octave configurations.

- Mount each blank on the lathe between centers. Turn the OD to body_OD_round (matching the body). Add the tenon at one end - diameter = bore_ID - drone_tenon_clearance, length = drone_tenon_len.

- Drill the drone bore axially (use a long drill bit chucked in the tailstock). Depth and ID per Excel Design_Table drone_L_physical and drone_bore_ID for the matching key + drone offset.

- For the unison block: drone bore matches melody fundamental - bore is the longest of the three.

- For the fifth block: drone is 7 semitones below fundamental - bore is shorter.

- For the octave block: drone is 12 semitones below fundamental - bore is shortest.

- Sand each drone block to 320 grit. Apply finish (Phase 11) before final fit-test.

- Engrave a small mark on the foot of each drone block with the laser cutter: U for unison, 5 for fifth, 8 for octave.

Phase 10: Bird Block

- Cut bird block blanks from the offcut walnut: 2 in long, 0.75 in tall, body_OD_flat wide.

- Use the laser cutter to engrave the totem pattern (eagle, wolf, bear, or geometric per flute personality).

- Cut a flat shoulder on the bottom of each bird block to match the top facet of the body. The shoulder must align over the window.

- Test position the bird block over the window. The flue gap is set by the bottom-edge profile of the bird - typical 0.030-0.050 in.

- If gap is too small, sand the bottom of the bird. If too large, glue a thin shim of veneer to the bottom.

Phase 11: Finish

- Final sand the entire body, drone blocks, and bird blocks: 220, 320, 400, 600 grit.

- Wipe down with a tack cloth.

- Apply Tru-Oil with a foam applicator. Thin first coat. Let dry 4-6 hours.

- Sand with 600 grit between coats.

- Apply 4-6 coats total. Final coat should be glassy.

- Wait 7 days for full cure.

- Buff with carnauba wax on a soft cloth. The flute should feel like silk.

- Reapply wax every few months for long-term care.

Phase 12: Final Tuning & QA

- Tie the bird block to the body with the leather lacing.

- Insert the unison drone block first.

- Play each note of the pentatonic minor scale. Use a chromatic tuner. Compare to Hole_Schedule!Hz columns.

- If a note is sharp, deepen the hole (file out the wall slightly). If flat, partially fill the hole edge with thin CA.

- Swap to the fifth drone block, then octave. Confirm each plays its expected drone pitch.

- Record a video of all four flutes being played in sequence. Archive to /Drone-Flutes/qa/ folder.

- Photograph each finished flute against a neutral background for the gallery.

# 6. Common Pitfalls & Mitigations

- Bore mismatch at glue-up: caused by datum pin slop. Mitigation: use 3/16 DOWELS not metal pins, slightly oversize so the pins crush-fit.

- Faceting tearout on grain reversal: feed direction matters. Mitigation: always cut walnut with the grain on the FACET pass; slow feed if you hear chatter.

- Window splitting edge too thin (collapses): caused by 1/16 upcut over-engaging. Mitigation: leave 0.005 in skin, finish with chisel.

- Inlay plug too loose: pocket cut wide due to bit deflection. Mitigation: do a test pocket on scrap first; adjust offset.

- Drone block too tight in socket: humidity. Mitigation: turn drone tenons in winter when wood is dry. Sand to fit if needed.

- Tuning sharp on hole 6 (octave): caused by hole too close to window. Mitigation: file the foot-side edge of the hole to effectively shift the hole away from the window.

# 7. Schedule

Suggested project timeline (calendar days). Many phases overlap; this is the critical path.

| Week | Phase(s) | Key Outputs |
| --- | --- | --- |
| Week 1 | Phase 0 (CAD), Phase 1 (stock acclimation) | G-code generated; blanks acclimating |
| Week 2 | Phase 2-3 (split-body CNC + glue-up) | Bore-profile bodies glued; 24h cure |
| Week 3 | Phase 4-7 (lathe, facets, holes, window) | All 4 bodies basically playable |
| Week 4 | Phase 8 (inlay) | Inlay complete on all bodies |
| Week 5 | Phase 9-10 (drone blocks + birds) | Block sets and birds finished |
| Week 6 | Phase 11 (finish) | Tru-Oil applied; cure time begins |
| Week 7 | Cure (no shop work) | Wait for finish to cure |
| Week 8 | Phase 12 (tuning + QA) | Photos, recordings, final inspection |

# 8. Sign-off

This BOM and build method document is the master shop reference. The Excel design workbook (Drone-Flutes-Design.xlsx) is the source of truth for all numerical dimensions. The SolidWorks reference doc (Drone-Flutes-SolidWorks-MasterLayout.docx) is the master CAD setup guide.

Together these three documents fully specify the build.