Native American Drone Flute

SolidWorks Master Layout Reference

F#m | Em | Dm | Am  -  Walnut + Maple  -  Hybrid CNC + Lathe Construction

# 1. Purpose

This document is the single reference for setting up the parametric SolidWorks model that produces all four drone flutes (F#m, Em, Dm, Am) from one master file. The model is driven by global variables and equations whose names exactly match the Excel workbook Drone-Flutes-Design.xlsx, so changing any input in either system can be propagated to the other through the design table link.

The companion files used together with this reference:

- Drone-Flutes-Design.xlsx -- the source of truth for all dimensions and configurations.

- Drone-Flutes-BOM-Build-Method.docx -- the shop instructions, materials, and CNC operations.

- inlay-patterns/ -- DXF and SVG files imported into the relevant inlay sketches.

# 2. Architecture Overview

Tony's preferred parametric pattern is the master sketch + design table. The master sketch sits on the Front plane and contains every dimension the rest of the model needs. Configurations are switched by changing key_midi (and a small number of dependent variables) through the design table. The design table is linked to the Excel file.

Key principles:

- Every numeric value in the model is either a global variable or derived from one through an equation. There are NO hand-typed dimensions inside features.

- Naming follows snake_case with the same identifiers used in Excel. Tony's pinned naming list lives in Section 3.

- Configurations are PER KEY only (Am, F#m, Em, Dm). Inlay variants are NOT configurations -- they live in saved-as copies because they affect cut sketches, which design tables cannot toggle cleanly.

# 3. Global Variables Reference

Open Tools > Equations > Manage Equations and add every row below as a Global Variable. Equations referencing other variables go in the Equations tab in dependency order. Names MUST match Excel exactly.

| Name | Value or Equation | Units | Notes |
| --- | --- | --- | --- |
| c_speed_of_sound | 13552 | in/s | Speed of sound at 68F. Same as Excel Master_Inputs!B7. |
| T_shop_temp_F | 68 | F | Shop reference temperature. |
| A4_reference | 440 | Hz | Concert pitch. Set 432 for alternative tuning. |
| end_correction_k | 0.6 | - | Open end correction multiplier. |
| SAC_length | 5.5 | in | Slow Air Chamber length. |
| SAC_wall | 0.25 | in | Wall between SAC and main bore. |
| flue_gap | 0.040 | in | Flue/window gap height. |
| bird_length | 2.0 | in | Totem block length. |
| bird_height | 0.75 | in | Totem block height above OD. |
| window_length | 0.625 | in | Sound window opening length. |
| window_width | 0.625 | in | Sound window opening width. |
| drone_offset_st | 0 | st | Drone offset semitones. 0=unison, -7=fifth, -12=octave. |
| drone_block_length | 3.5 | in | Length of swappable drone block. |
| drone_tenon_len | 0.75 | in | Tenon engagement length. |
| drone_tenon_clear | 0.005 | in | Slip-fit clearance on drone tenon. |
| n_facets | 8 | count | Number of body facets (octagon). |
| facet_chord | TAN(PI/n_facets) | - | Calculated factor: side = OD * facet_chord. |
| facet_inset | 0.10 | in | Inlay inset from facet edge. |
| facet_allowance | 0.05 | in | Extra material left after lathe rounding. |
| key_midi | 69 | midi | A4=69, F#4=66, E4=64, D4=62. Drives the active configuration. |
| fundamental_freq | A4_reference * 2^((key_midi-69)/12) | Hz | Computed fundamental. |
| drone_freq | fundamental_freq * 2^(drone_offset_st/12) | Hz | Drone fundamental. |
| bore_ID | 0.875 | in | Per-config (changes via design table). |
| wall_thickness | 0.25 | in | Bore wall thickness. |
| body_OD_round | bore_ID + 2*wall_thickness + facet_allowance | in | Round blank diameter before faceting. |
| body_OD_flat | body_OD_round - 2*facet_allowance | in | Flat-to-flat after faceting. |
| facet_side | body_OD_flat * facet_chord | in | Length of one facet edge. |
| K2_correction | lookup table by bore_ID | % | Empirical bore-size correction. |
| L_acoustic_raw | c_speed_of_sound / (2*fundamental_freq) | in | Open-open pipe formula. |
| L_acoustic_corr | L_acoustic_raw * (1 + K2_correction) | in | After K2 empirical correction. |
| L_bore_phys | L_acoustic_corr - 2*end_correction_k*bore_ID/2 | in | Physical bore length. |
| L_total | L_bore_phys + SAC_length + SAC_wall + bird_length - window_length | in | Total flute length. |
| hole1_pos | L_acoustic_corr * (1 - fundamental_freq/(fundamental_freq*2^(3/12))) | in | m3 hole, distance from foot. |
| hole2_pos | L_acoustic_corr * (1 - fundamental_freq/(fundamental_freq*2^(5/12))) | in | P4 hole. |
| hole3_pos | L_acoustic_corr * (1 - fundamental_freq/(fundamental_freq*2^(7/12))) | in | P5 hole. |
| hole4_pos | L_acoustic_corr * (1 - fundamental_freq/(fundamental_freq*2^(8/12))) | in | m6 hole. |
| hole5_pos | L_acoustic_corr * (1 - fundamental_freq/(fundamental_freq*2^(10/12))) | in | m7 hole. |
| hole6_pos | L_acoustic_corr * (1 - fundamental_freq/(fundamental_freq*2^(12/12))) | in | octave hole. |

Variable list - 38 total. Same identifiers appear in Excel Master_Inputs sheet. Edit in only one place; design table propagates.

# 4. Configuration Design Table

Insert > Tables > Design Table > From Existing File. Select Drone-Flutes-Design.xlsx and the Design_Table sheet. Tell SolidWorks to use the column labeled Configuration as the configuration name and the rest as parameters. Recommended starter columns:

| Configuration | key_midi | bore_ID | wall_thickness | drone_bore_ID |
| --- | --- | --- | --- | --- |
| Am | 69 | 0.750 | 0.25 | 0.625 |
| F#m | 66 | 0.875 | 0.25 | 0.750 |
| Em | 64 | 1.000 | 0.25 | 0.875 |
| Dm | 62 | 1.125 | 0.25 | 1.000 |

Add additional columns as needed for inlay parameters or bird/window variants. SolidWorks will create one configuration per row.

Tip: when SolidWorks asks whether to update the model from Excel, choose Update Excel. This way edits made inside SolidWorks during shop tweaking flow back to the workbook.

# 5. Sketch Hierarchy

Build sketches in this order to keep the dependency graph clean. Later sketches reference earlier ones via convert-entities and pierce relations to the master profile.

| Sketch | Plane / Face | Purpose |
| --- | --- | --- |
| Sketch_Master_Profile | Front | Top-level layout: every other sketch references this. Contains the L_total reference line, foot point at 0, and key landmarks (foot, bore_start, window, SAC_start, bird_top, drone_socket). |
| Sketch_Bore_Profile | Front | The interior bore as a 2D profile. Driven by L_acoustic_corr, bore_ID, SAC_length, SAC_wall. |
| Sketch_Hole_Centers | Top | 6 driven points along the body for the 6 finger holes; X driven by hole1_pos through hole6_pos. |
| Sketch_Window_Profile | Top | TSH window cutout: window_length x window_width rectangle with bird_height clearance shelf. |
| Sketch_Octagon_Section | Right | Cross-section with 8 sides. Inscribed in body_OD_flat circle. Drives all facet operations. |
| Sketch_Drone_Tenon | Front | Drone block tenon profile. drone_tenon_len, (bore_ID - drone_tenon_clear) diameter. |
| Sketch_Drone_Bore | Front | Drone interior cavity. Independent acoustic chamber driven by drone_freq. |
| Sketch_Inlay_Band1 | Octagon facet faces | Lowest geometric band (15% from foot). Wraps all 8 facets. |
| Sketch_Inlay_Band2 | Octagon facet faces | Center band (50%). |
| Sketch_Inlay_Band3 | Octagon facet faces | Upper band (82%, just below window). |
| Sketch_Feature_Panel | Single facet | Larger feature inlay between band 2 and band 3. |

Sketch dependency tree. Master_Profile is the root; everything else pierces or converts from it.

# 6. Suggested Feature Tree Order

This is the recommended top-to-bottom order in the FeatureManager. It keeps geometry valid through all four configurations and avoids circular sketch references.

01  Sketch_Master_Profile

02  Reference Plane: Foot_Plane (offset along master profile)

03  Reference Plane: Window_Plane

04  Reference Plane: SAC_Splitting_Plane

05  Sketch_Octagon_Section  (driven by body_OD_flat)

06  Boss-Extrude  Body_Round  (length = L_total, diameter = body_OD_round)  [for blank docs]

07  Cut-Extrude   Octagon_Facet_Pass  (8 sides)

08  Sketch_Bore_Profile

09  Cut-Revolve   Main_Bore  (driven by L_acoustic_corr)

10  Sketch_Window_Profile

11  Cut-Extrude   Window_Cutout

12  Sketch_Hole_Centers

13  Cut-Extrude   Finger_Holes_x6

14  Sketch_Drone_Bore

15  Cut-Revolve   Drone_Cavity  (separate body)

16  Boss-Extrude  Bird_Block  (saved out as separate part for laser cutting)

17  Sketch_Inlay_Band1 / Band2 / Band3 / Feature_Panel

18  Cut-Extrude   Inlay_Pockets  (depth = band_inlay_depth)

19  Configuration tab: Am, F#m, Em, Dm  (driven by Excel design table)

# 7. Excel <-> SolidWorks Cross-References

These bindings are the load-bearing connections. If you rename anything on either side, both ends must update. The Excel Master_Inputs cells are the canonical home of each value.

| Excel Cell | SolidWorks Variable | Notes |
| --- | --- | --- |
| Master_Inputs!B7 | c_speed_of_sound | Excel = SolidWorks. Both at 13552 in/s. |
| Master_Inputs!B9 | A4_reference | Concert pitch. Edit in either, sync the other. |
| Master_Inputs!B22 | drone_offset_st | Drone tuning offset. Drives drone_freq. |
| Master_Inputs!B29 | n_facets | Octagon by default. Sketch_Octagon_Section is parametrically built around this. |
| Design_Table!C12-F12 | bore_ID | Per-key bore. Excel design table maps to SolidWorks design table rows. |
| Design_Table!C8-F8 | fundamental_freq | Computed in both, should match. |
| Hole_Schedule rows 7-12 cols D/G/J/M | hole1_pos through hole6_pos | Bidirectional check; SolidWorks reads from Excel via design table link. |

Bidirectional cross-reference table. SolidWorks reads from Excel via the design table; Excel can also receive updates through Update Excel during edits.

# 8. Build Recipe

Step-by-step procedure for setting up the model from a blank part. Should take about 60-90 minutes the first time, faster on subsequent rebuilds.

- Open SolidWorks. File > New > Part. Set units to inches.

- Tools > Equations > Manage Equations. Switch to Global Variables tab.

- Add every variable from Section 3 Global Variables Reference. Use the EXACT names shown so Excel sync works.

- Switch to Equations tab. Add the 12 derived equations (fundamental_freq, L_acoustic_corr, etc.).

- For K2_correction: SolidWorks does not support lookup tables in equations. Use IIF cascade: IIF(bore_ID < 0.75, -0.020, IIF(bore_ID < 0.875, -0.007, IIF(bore_ID < 1.0, 0.004, IIF(bore_ID < 1.125, 0.007, 0.013)))).

- Front plane: insert Sketch_Master_Profile. Place a horizontal construction line of length L_total. Foot at origin.

- On the master profile add labeled points: foot (0), bore_start (L_total - SAC_length - bird_length + window_length), window_center, SAC_splitting_edge, drone_socket_top.

- Create reference planes offset from Front using these points.

- Right plane: insert Sketch_Octagon_Section. Polygon tool, 8 sides, inscribed circle = body_OD_flat / 2. Make construction.

- Boss-Extrude the octagon along the master profile to make the body. Length = L_total.

- Sketch_Bore_Profile on Front: rectangle (or revolve profile) driven by bore_ID and L_bore_phys. Cut-Revolve.

- Sketch_Window_Profile on the top facet: window_length x window_width.

- Sketch_Hole_Centers along the top facet: six points driven by hole1_pos through hole6_pos. Cut-Extrude through.

- Repeat the bore process for the drone chamber on a parallel offset plane.

- Insert > Tables > Design Table. SolidWorks will detect global variables. Map the columns shown in Section 4.

- Link the design table to the Excel Drone-Flutes-Design.xlsx file via Excel Source on the Design Table dialog. Choose the Design_Table sheet, range C5:F40.

- Test by switching configurations. Confirm body, bore, and hole positions all update.

- For inlay: open separate sketches on each facet (use Edit Sketch > set sketch plane to facet face). Sketch geometric bands using construction-line offsets driven by band1_pct, band2_pct, band3_pct.

- Save the part as DroneFlute_Master.SLDPRT. Save four configurations into a single design table.

- For CNC, use Save As > DXF/DWG of each sketch (window, holes, inlay) and STL of the bore-cut profile.

# 9. Inlay Sketch Strategy

The geometric bands and feature panels are imported as DXF into facet-face sketches. Each band has its own sketch so it can be toggled per configuration if you want different patterns per key.

- Open the desired facet face. Insert > DXF/DWG > Wrap. Place at the band y-position from Inlay_Layout!C9-C11 (or 1-1 link).

- Cut-Extrude with depth = band_inlay_depth (default 0.060 in for narrow bands, 0.080 in for the feature panel).

- For the plug pattern, the same DXF is used scaled by 1 + plug_oversize/2 inside the geometry, so a 0.002 in interference is generated at glue-up.

- Save both pocket and plug DXFs to the inlay-patterns/ folder under names matching the band, e.g. F#m_band1_pocket.dxf and F#m_band1_plug.dxf.

# 10. Verification Checklist

- Switch through all four configurations. Confirm L_total updates and the body length changes accordingly.

- Open Mass Properties on the body for each configuration. Confirm volume and surface area are within reasonable bounds.

- Cross-check fundamental_freq against Design_Table!C8-F8. Should match Excel within rounding.

- Cross-check hole1_pos through hole6_pos against Hole_Schedule!*7-*12. Differences > 0.005 in indicate K2 or end-correction divergence.

- Save a flat-pattern DXF of each facet (Bend > Insert > Sheet Metal-style flatten will not work; instead use Save As DXF on each facet face individually) and confirm dimensions match Inlay_Layout!facet_side_length.

- For drone block: open the drone chamber separately, confirm drone_freq matches Design_Table!*9 for the active configuration.

# 11. Known Edge Cases & Tips

- SolidWorks 2024 onward truncates global variable names at 32 characters. Several names here are at 18-22 chars to stay safe.

- Linking a design table to a NETWORK Excel file (OneDrive sync) sometimes triggers permission errors. If this happens, copy the Excel file to a local path before linking.

- If Excel changes do not propagate, verify Tools > Options > External References > Update External References on Open is set to Always.

- The drone chamber is parameterized as a SEPARATE body in the same part. Use Insert > Boss > Combine to merge for solid body operations only after all features are added.

- Inlay DXFs use the bottom-left corner as the import origin. Keep all DXF files anchored at (0,0) so they import in the right spot every time.

- The window cutout is intentionally NOT extruded through the SAC; the splitting edge is generated by an offset surface. Avoid Convert Entities here -- use Sketch + Cut-Extrude with explicit numeric depth = SAC_wall.

# 12. Files & Naming Convention

| File | Purpose |
| --- | --- |
| DroneFlute_Master.SLDPRT | Single part with 4 configurations and the linked design table. |
| DroneFlute_Bird.SLDPRT | Totem block as a separate part. One geometry, multiple inlay variants. |
| DroneFlute_DroneBlock.SLDPRT | Tunable drone block part with 3 configurations (unison, fifth, octave). |
| DroneFlute_Master.SLDDRW | Working drawing with views needed by Tony in the shop. |
| Drone-Flutes-Design.xlsx | External Excel file linked into design table. |
| inlay-patterns/*.dxf | Inlay artwork imported into facet sketches. |

Keep all CAD files together in /Drone-Flutes/sw-reference/ alongside this document.